# HyForce Project Documentation

Hytale QUIC Protocol Analysis and Decryption Research

Date: March 5, 2026 | Status: Work in Progress

# 1. Executive Summary

This document provides a comprehensive overview of the HyForce project - a Hytale game packet interception and decryption tool. The project aims to capture, analyze, and decrypt QUIC protocol traffic between the Hytale client and server.

## 1.1 Current Status

The project has made significant progress in understanding the Hytale network protocol but is currently facing challenges with decryption. Key achievements include:

• Successfully capturing QUIC packets from Hytale client

• Implementing SSLKEYLOGFILE parsing for TLS 1.3 secrets

• Developing HKDF key derivation for QUIC keys

• Creating a comprehensive debug logging system

• Building an ImGui-based user interface

## 1.2 Current Challenges

The primary challenge is that decryption attempts consistently fail with Authentication tag mismatch errors. Despite having valid TLS secrets from SSLKEYLOGFILE, the derived keys do not successfully decrypt captured packets.

# 2. Hytale Protocol Overview

## 2.1 Transport Protocol

Hytale uses QUIC (Quick UDP Internet Connections) as its transport protocol, running over UDP on port 5520 by default. This is a significant departure from Minecraft's TCP-based protocol.

Key QUIC Characteristics:

• Built-in TLS 1.3 encryption

• Faster connection establishment (0-RTT and 1-RTT)

• Better handling of packet loss

• Multiplexed streams without head-of-line blocking

• Connection migration support

## 2.2 Netty Implementation

Hytale uses Netty's QUIC codec (netty-incubator-codec-quic) for handling QUIC connections. The server implementation is built on Java 25.

Key Components:

- netty-incubator-codec-quic: QUIC protocol implementation

- BoringSSL: TLS/QUIC cryptographic operations

- Zstd compression: Packet payload compression

- VarInt encoding: Variable-length integer fields

## 2.3 Packet Structure

Hytale packets use the following wire format:

[4 bytes] Length (little-endian) - Payload size

[4 bytes] Packet ID (little-endian)

[...] Payload (may be Zstd-compressed)

Key Characteristics:

- Byte Order: Little-endian throughout

- Compression: Zstd for large packets

- VarInt: 7-bit encoding, max 5 bytes

- Strings: UTF-8 with VarInt length prefix

- Max Payload: ~1.56GB (0x64000000 bytes)

# 3. QUIC Protocol Deep Dive

## 3.1 QUIC Header Types

QUIC has two header types: Long Header (used during handshake) and Short Header (used after connection establishment).

Long Header Format:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|1| T | S | R |R|R|R|R|                Version                  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| DCID Len (8)  |             Destination Connection ID         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| SCID Len (8)  |               Source Connection ID            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                           Payload                             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

## 3.2 TLS 1.3 Key Derivation

QUIC uses TLS 1.3 for key exchange. The key derivation follows RFC 8446 using HKDF-Expand-Label.

HKDF-Expand-Label(Secret, Label, Context, Length) =

HKDF-Expand(Secret, HkdfLabel, Length)

Where HkdfLabel is:

struct {

uint16 length = Length;

opaque label<7..255> = "tls13 " + Label;

opaque context<0..255> = Context;

} HkdfLabel;

## 3.3 QUIC Key Derivation (RFC 9001)

QUIC derives three keys from each traffic secret:

From client_traffic_secret_0:

- quic key (16 bytes for AES-128-GCM, 32 for AES-256-GCM)

- quic iv (12 bytes)

- quic hp (16 bytes for header protection)

From server_traffic_secret_0:

- quic key

- quic iv

- quic hp

Label format: No "tls13 " prefix for QUIC (RFC 9001)

# 4. Current Implementation Status

## 4.1 Project Structure

HyForce/

├── Core/

│   ├── AppState.cs           - Application state management

│   ├── PacketDecryptor.cs    - QUIC decryption engine

│   └── DecryptionDebug.cs    - Debug logging system

├── Tabs/

│   ├── DecryptionTab.cs      - Decryption UI

│   ├── PacketTab.cs          - Packet capture UI

│   └── SettingsTab.cs        - Settings UI

├── Protocol/

│   ├── PacketHandler.cs      - Packet processing

│   └── HytaleProtocol.cs     - Protocol definitions

└── Data/

├── CapturedPacket.cs     - Packet data structure

└── PacketLog.cs          - Packet storage

## 4.2 Key Classes and Methods

PacketDecryptor (Static Class):

public static class PacketDecryptor

{

// Properties

public static bool DebugMode { get; set; }

public static bool AutoDecryptEnabled { get; set; }

public static int MaxDCIDLengthToTry { get; set; } = 5;

public static int DecryptionTimeoutMs { get; set; } = 100;

// Methods

public static void AddKey(EncryptionKey key)

public static void DeriveQUICKeys(EncryptionKey key)

public static byte[] TryDecrypt(byte[] packet, EncryptionKey key, int dcidLength)

public static DecryptionResult TryDecrypt(byte[] packet)

public static DecryptionResult TryDecryptManual(byte[] packet, int timeoutMs)

public static void LoadSSLKeyLog(string filePath)

public static string TestKeyDerivation()

public static string DumpAllKeys()

}

## 4.3 Current Debug Features

• Console logging with color-coded output

• File logging to decryption_debug.log

• In-game log panel with packet activity

• Key derivation testing

• Manual decryption test lab

• SSL key log loading

• Connection tracking

# 5. Known Issues and Debugging

## 5.1 Decryption Failure Pattern

The current implementation consistently fails with Authentication tag mismatch errors. This indicates that while keys are being derived correctly, they do not match the packet encryption keys.

Possible Causes:

• Wrong DCID length - The DCID length used for key derivation doesn't match the actual packet

• Wrong label format - RFC 9001 vs RFC 8446 label format mismatch

• Wrong cipher - AES-128-GCM vs AES-256-GCM mismatch

• Key/Packet mismatch - SSL keys from different connection than captured packets

• Header protection issues - Incorrect HP key usage

• Nonce construction errors - Wrong packet number encoding

## 5.2 Menu Freezing Issue

The menu freezes when opening the Decryption tab. This is caused by:

• Synchronous decryption attempts blocking the UI thread

• Too many DCID lengths being tried (0-20)

• No timeout on decryption operations

• Excessive debug logging

## 5.3 SSL Key Log Analysis

The user's sslkeys_permanent.log contains 26+ connections, each with unique secrets. This is expected behavior when resetting the proxy multiple times.

Sample SSLKEYLOGFILE format:

CLIENT_TRAFFIC_SECRET_0 <client_random> <secret_hex>

SERVER_TRAFFIC_SECRET_0 <client_random> <secret_hex>

Example:

CLIENT_TRAFFIC_SECRET_0 a703983e3165853ec056f5ad2d541b0164711a6da517c33e5d1d942aea76081d 71c7676ac4a0ed15206e7728420eca39ca326f4a6b55e82992f792c617e009fb

# 6. Reference Materials

## 6.1 Official Documentation

• RFC 9000: QUIC - A UDP-Based Multiplexed and Secure Transport

• RFC 9001: Using TLS to Secure QUIC

• RFC 8446: The Transport Layer Security (TLS) Protocol Version 1.3

• RFC 5869: HMAC-based Extract-and-Expand Key Derivation Function (HKDF)

## 6.2 Hytale-Specific Resources

• Hytale Server Manual: https://support.hytale.com/hc/en-us/articles/45326769420827

• Hytale Protocol Docs (Unofficial): https://hytale-docs.pages.dev/modding/networking/protocol/

• Netty QUIC Codec: https://netty.io/news/2026/02/17/quic-0-0-75-Final.html

• Hytale Protocol GitHub: https://github.com/topics/hytale-protocol

## 6.3 QUIC Decryption Resources

• Wireshark QUIC Decryption: https://wiki.wireshark.org/QUIC

• SSLKEYLOGFILE Format: https://developer.mozilla.org/en-US/docs/Mozilla/Projects/NSS/Key_Log_Format

• QUIC Test Vectors: https://github.com/quicwg/base-drafts/wiki/Test-Vector-for-the-Initial-AEAD-key-derivation

# 7. Recommended Next Steps

## 7.1 Immediate Actions

• Verify SSL keys work with Wireshark first

• Test with a single connection's keys only

• Try different DCID lengths systematically

• Compare derived keys with known test vectors

## 7.2 Code Improvements

• Add async/await for all decryption operations

• Implement proper cancellation tokens

• Add packet capture to file for offline analysis

• Create Wireshark-compatible key export

• Add packet structure analysis tools

## 7.3 Testing Strategy

• Use Wireshark to verify SSL keys are correct

• Test with known QUIC implementations (quiche, ngtcp2)

• Compare packet captures with successful decryptions

• Verify DCID extraction from captured packets

# 8. Technical Implementation Notes

## 8.1 HKDF Label Formats Tested

Format 1 - RFC 9001 (Standard QUIC):

Label: "quic key"

Result: Keys derived successfully

Format 2 - RFC 8446 (TLS 1.3 style):

Label: "tls13 quic key"

Result: Keys derived successfully

Format 3 - QUICv2:

Label: "quicv2 key"

Result: Keys derived successfully

Format 4 - Test Vector:

Uses RFC 9001 test vectors for verification

## 8.2 Key Derivation Verification

All label formats successfully derive keys from the TLS secrets. The issue is not with key derivation but with key usage during decryption.

## 8.3 Connection Tracking

The current implementation tracks connections by hashing the secret. This may not match Hytale's connection ID scheme.

Current approach:

string connId = SHA256(secret).Take(16).ToHex()

Potential issues:

- Hytale may use different connection ID generation

- Multiple secrets may belong to same connection

- Connection ID may be derived differently

# 9. Conclusion

The HyForce project has made significant progress in understanding Hytale's QUIC-based protocol. The key challenges remaining are:

• Successfully decrypting captured packets with derived keys

• Resolving the menu freezing issue

• Verifying the correct DCID length and label format

With continued debugging and systematic testing, particularly using Wireshark for verification, the decryption issue should be resolvable.

Document compiled from research and development conducted March 2026.